Practical Analytical 1 Chemistry

Experimental 5: 
Corrosion of Iron

*Chemicals and tools:

- Four Flasks

- Nails

- Water

- Pre-boiled water

- Salt water

- Drying agent (CaCl2)

Procedure:

- Put the first nail in Flask 1 containing aerated water

- Put the second nail in Flask 2 containing drying agent without water and close it firmly

- Put the third nail in Flask 3 containing deaerated tap OR distilled water (Pre-boiling water) and close it firmly

- Put the nail in Flask 4 containing aerated salty water and close it firmly

- Leave all Flasks for one week and record your observations.

Results

1-Air + Water (aerated tap OR distilled water)

2-Dry agent (CaCl2 powder)

3-Pre boiling tap OR distilled water (Deaerated water)

4-Aerated salty water

Comparison between the 4 flasks

Conclusions

- Rusting is destruction of a metal due to the contact with its surrounding environment.

- Electrochemical corrosion takes place at the anodes of the micro cells formed on metal surface.

- The most corrosive environment are:

  - Air (moist and humid, hot, polluted),

  - Water (hot, salty, polluted).

- The nail in the dry air (Flask 2) does not corroded.

- The nail in the aerated salty water (Flask 4) is the most corroded one.

- The corrosion rate increases in the order:

- (Flask 2) < (Flask 3) < (Flask 1) < (Flask 4)

| Dry air | < deaerated water | < aerated water | < aerated salty water |
| --- | --- | --- | --- |